# Chapter 1

## Introduction

This mainboard has a slot-1 processor socket for an Intel processor cartridge, and it also has a socket-370 for an Intel PPGA (Plastic Pin Grid Array) Celeron processor. You can install either one of these processors according to the power and performance requirements that you need from your system. Note that you cannot install two processors on this mainboard.

Slot-1 processors include the SEPP (Single Edge Processor Package) Celeron, the Pentium-II, and the Pentium-III. Clock rates run as high as 600 MHz. Socket-370 processors are the PPGA Celeron which runs at clock rates up to 533 MHz. The mainboard supports a system bus of 66 MHz or 100 MHz.

The mainboard uses the BXcel chipset which is highly integrated and includes a built-in PCI 3D Sound System. A V.90 Fax/ Modem DAA module is also shipped with the mainboard. In addition, the mainboard has a full set of I/O Ports including two serial ports, a parallel port, one PS/2 port and two USB ports.

This mainboard has all the features you need to develop a powerful multimedia workstation with built-in communications. The board is baby-AT sized and has power connectors for ATX and AT power supply units.

### Key Features

This key features of this mainboard include:

#### Slot-1 Processor Support

- Pentium-III support for 450 MHz to 600 MHz clock rates

- Pentium-II support for 233 MHz to 450 MHz clock rates

- SEPP Celeron support for 266 MHz to 433 MHz clock rates

- Support for 66 MHz and 100 MHz FSB (Front Side Bus)

#### Socket-370 Processor Support

- The PPGA Celeron provides Pentium-II performance with integrated level 1 and level 2 cache memory

- PPGA Celerons run from 300 MHz to 533 MHz.

- Supports a 66 MHz front side system bus

#### Memory Support

- Three DIMM slots for SDRAM/EDO/FPM 168-pin memory modules

- Support for 66 MHz & 100 MHz memory bus

- Maximum installed memory can be 3 x 256 MB = 768 MB.

#### Expansion Slots

- One AGP Slot

- Three 32-bit PCI slots

- One 8/16-bit ISA slot

#### Onboard IDE channels

- Primary and Secondary PCI IDE channels

- Support for PIO (programmable input/output) modes

- Support for Bus mastering and UltraDMA 33 modes

#### Power Supply and Power Management

- Dual connector for either AT or ATX power supply

- Support for Green PC standard, suspend switch

- Support Wake on Modem and Wake on Alarm

#### Sound System

- Meets PC98 audio specification

- Full duplex playback and recording with built-in 16-bit CODEC

- HRTF 3D professional audio supports both Direct Sound 3D® and A3D® compatible interface plus support for 4-channel speakers

- Drivers support DOS/Windows 95/98/2000/NT 4.0

- Built-in 32 ohm earphone buffer and 3D surround

- Provides MPU-401 Game/MIDI port and legacy Sound Blaster 16 support

- Downloadable Wave-table Synthesizer supports Direct Music®

- Optional Digital Audio Interface (SPDIF In/Out) with 24-bit stereo, 44KHz sampling rate and measured 120dB audio quality

- Optional optic fibre interface which enables communication with MiniDisk or high-end audio systems

- Stereo Mixer supports analog mixing from CD-Audio, Line-In, and digital mixing from voice, FM/Wave-table and digital CD-Audio

#### Onboard I/O Ports

- Floppy disk drive port with 1Mb/s transfer rate

- Two serial ports with 16550-compatible fast UART

- One parallel port with support for ECP and EPP

- Two USB ports, one PS/2 mouse port and one infrared port (optional)

#### Hardware Monitoring

- Built-in hardware monitoring for CPU/system temperatures, fan speeds and mainboard voltages

- Supports AMI’s  Desktop Client Manager (ADCM)

#### Fax/Modem DAA Module

- 56 Kbps Fax/Modem DAA module

- Supports V.90, V.34, V.32bis, V.32, V.22bis, V.22

- Supports Auto Fallback and MNP 5, V.42bis data compression with 115200 compatible Virtual UART

- Requires 16 MB RAM and WIN 95/98/NT

#### Onboard Flash ROM

- Supports plug and play configuration of peripheral devices and expansion cards

- Built-in virus protection using Trend’s ChipAwayVirus which ensures that the entire boot process is virus protected.

#### Bundled Software

- AMI Desktop Client Manager supports hardware monitoring on stand alone systems or over a network

- PC-Cillin provides automatic virus protection under Windows 95/98

- SuperVoice is Fax/Modem software with support for data and voice transmission

- MediaRing Talk is an internet telephone application.

- Gamut is an audio application that includes MP3 encoding/decoding

- WordPerfect Suite 8 is a windows version office application

#### Dimensions

- Baby-AT form factor (22cm x 24cm)

### Package Contents

Your mainboard package ships with the following items:

- Mainboard

- This User’s guide

- IDE cable

- Floppy diskette drive cable

- Audio ports and Game/MIDI port extension bracket

- Serial ports extension bracket

- Parallel port extension bracket

- V.90 Fax/Modem DAA module

- Support software CD-ROM

#### Optional Accessories

You can purchase the following optional accessories for this mainboard.

- ATX Form Card (2 USB ports, IR port & PS/2 port)

- Optic module extension bracket

### Static Electricity Precautions

- Components on this mainboard can be damaged by static electricity. Take the following precautions when unpacking the mainboard and installing it in a system.

- Keep the mainboard, and other components, in their original static-proof packaging until you are ready to install them.

- During an installation, wear a grounded wrist strap if possible. If you don’t have a wrist strap, frequently discharge any static electricity by touching the bare metal of the system chassis.

- Handle the mainboard carefully by the edges. Avoid touching the components unless it is absolutely necessary. During the installation lay the mainboard on top of the static-proof packaging with the component side facing upwards.

- Inspect the mainboard for any damage caused during transit. Ensure that all the components that are plugged into sockets are correctly seated.

- If you suspect that the mainboard has been damaged, do not apply power to the system. Contact your mainboard vendor and report the damage.