Addendum Page for 
M560 TXPRO Chipset

Version 4.1 Mainboard

M560 (ver 4.1) Mainboard Component Locations

TX PRO

82C371

COM1

COM2

AMI BIOS

RTC

Power

Connector

PS1

JP15:

CMOS Discharge

SIMM

Socket

SIM4  SIM3  SIM2  SIM1

FDC

PRN 1

USB

PCI SLOT

ISA SLOT

IDE2

DIMM Socket

IDE1

DIMM2 DIMM1

3V

JP4

5V

TX PRO

82C439

512K Pipeline Cache

Key Lock

SPK

RST

HDD-LED

TB-LED

3         1

P54C

JP10

P55C

A

B

C JP14

D

E

JP5

JP6

JP7

Quick Installation Guide

1.	Set JP1 to CMOS RAM discharge jumper (pin 1-2).

2.	Set JP3 to select CPU speed & JP9 to select CPU type.

3.	Set JP5 to select CPU Internal Clock Speed.

4.	Set JP6-A, B, C, D to select CPU Voltage Regulator Output.

5.	Insert CPU to CPU socket.

6.	Insert 72-pin SIMM module into SIMM1-4 and / or insert 168-pin DIMM modules into DIMM1-2.  Notice that DIMM2 and SIMM1,2 cannot be installed at the same time.

7.	Install mainboard into system chassis.

8.	Connect keyboard to J1.

9.	Install the display card and other peripheral cards (if required) on to the mainboard.

10.	Connect hard disk(s) to IDE primary/secondary connection(s).

11.	Connect floppy drives(s) to FDC1 connector.

12.	Connect serial ports to COM1 and COM2 connectors

13.	Connect parallel port to PRM1 connector.

14.	Connect J3-HDD LED to “Hard Disk Busy” LED on the system chassis.

15.	Connect J3-TB LED to Turbo LED on the system chassis.

16.	Connect J3-RST to Reset Switch on the system chassis.

17.	Connect J3-SPK to Speaker on the system chassis.

18.	Connect J3-KBYLOCK to keylock and power LED on the system chassis.

19.	Connect power cord to J2 Power Supply Connector.

20.	Close system chassis, connect all external cables to your computer.

Jumper Setting

CN1	Keyboard Connector

| Pin | Description |
| --- | --- |
| 1 | Keyboard Clock |
| 2 | Keyboard Data |
| 3 | N.C. |
| 4 | Ground |
| 5 | +5VDC |

J3	Power Supply Connector

| Pin | Description | Pin | Description |
| --- | --- | --- | --- |
| 1 | Power Good | 7 | Ground |
| 2 | +5VDC | 8 | Ground |
| 3 | +12VDC | 9 | -5VDC |
| 4 | -12VDC | 10 | +5VDC |
| 5 | Ground | 11 | +5VDC |
| 6 | Ground | 12 | +5VDC |

COM1/2 - Serial Port #1/#2

CN5 - Floppy Disk Port

CN6 - Parallel Port

CN8/CN7 - Primary/Secondary IDE Ports

JP15	CMOS RAM Discharge Jumper

| Setting | Description |
| --- | --- |
| OPEN | Normal Mode |
| SHORT | Discharge CMOS |

J2	Univeral Serial Bus Connector

| Pin | Description |
| --- | --- |
| 1,2 | +5VDC |
| 3,4 | Data - |
| 5,6 | Data+ |
| 7,8 | Ground |

JP3	DIMM Module Voltage Selector

| Description | JP3 |
| --- | --- |
| 1 A B 5V 3.3V For SDRAM DIMM Module (3.3V) |  |
| 1 A B 5V 3.3V For EDO DRAM/Fast Page DRAM DIMM Module (5V) |  |

CN2	PS/2 Mouse Connector

| Pin | Description |
| --- | --- |
| 1 | Mouse Data |
| 2 | N.C. |
| 3 | Ground |
| 4 | +5V |
| 5 | Mouse CLK |
| 6 | N.C. |

J3	RST (Reset Swich Connector)

| Setting | Description |
| --- | --- |
| Open | Normal Mode |
| Short | Reset System |

J6	KEYLOCK (Keylock & Power LED Connector)

| Pin | Description |
| --- | --- |
| 1 | LED Output |
| 2 | N.C. |
| 3 | Ground |
| 4 | Keylock |
| 5 | Ground |

J5	HDD LED (Hard Disk LED Connector)

| Setting | Description |
| --- | --- |
| Open | Normal Mode |
| Short | Reset System |

JP8	TB LED (Turbo LED Connector)

| Setting | Description |
| --- | --- |
| 1 | Anode (+) |
| 2 | Cathode (-), Ground |

J8	SPK (Speaker Connector)

| Pin | Description |
| --- | --- |
| 1 | Speaker Out |
| 2 | Ground |
| 3 | Ground |
| 4 | 5V |

JP5,6,7CPU Speed Jumpers

| External Clock | JP5, 6, 7 | CPU Clock | JP5, 6, 7 | CPU Clock | JP5, 6, 7 |
| --- | --- | --- | --- | --- | --- |
| 1 1 1 1 50MHz |  | 66MHz |  | 83MHz |  |
| 1 1 60MHz |  | 75MHz |  |  |  |

JP13,12CPU Internal Clock Speed Selectors

|  | JP13,12 |  | JP13,12 |
| --- | --- | --- | --- |
| 1.5X | OFF OFF | 2.5X | ON ON |
| 2.0X | ON OFF | 3.0X | OFF ON |

JP6	P54C/P55C CPU Voltage Selectors

|  | JP10 A B C D |  | JP10 A B C D |
| --- | --- | --- | --- |
| 3.5V |  | 2.8V |  |
| 3.2V |  | 2.5V |  |
| 2.9V |  |  |  |